Development - Photoresistor Evaluation

Experiment Description

D

C

B

A

- An experiment was designed to determine if a photo resistor (Light dependant Resistor (LDR)) could be used to:

  - Test for the presence of a bird sized object on a bird table

  - Differentiate between different sized objects (e.g. a robin and a pigeon )

- Two different sized MDF circles of radii 50mm and 100mm were constructed to represent different sized birds (A)

- The photo resistor circuit was constructed (B)

- The photo resistor was attached to a pillar such that its height could be adjusted to match the centre of each circle (C)

- A ruler was used to measure the distance from the circle to the sensor (D)

- Using each circle in turn:

  - The light intensity (resistance) reading was taken (this value was subtracted from other readings to give the change in light intensity

  - The circle was located at a distance of 300mm from the sensor (perpendicular to it)  and the change in light intensity calculated

  - The circle was moved towards the sensor in small steps, the reading taken, and the change in light intensity calculated (E/F)

E

F

Circuit Diagram & Software

Results

- The table and graph above show the results obtained

- The Light intensity (photo resistor resistance) changes were only detectable within a distance 90mm from the sensor

- The variation in light intensity from 90mm to 10mm was small

- No significant difference was found in light intensity change between circles of different size at a given distance

G

H

- The sensor appeared to be sensitive to light changes in all directions so a tube was attached to try and make it sensitive in one direction (G/H)

- The experiment was repeated but the results were the same